GNS3 ENTERPRISE NETWORK INFRASTRUCTURE LAB

Portfolio Documentation

Vasanth Kumar R

BCA Graduate | Bengaluru, Karnataka

vasanthkumarvk2855@gmail.com  |  github.com/vasanth-kumar-vk

linkedin.com/in/vasanth-kumar-vk55

Technologies Used:

GNS3  |  VMware Workstation Pro  |  pfSense CE  |  Cisco IOS

Windows Server 2022  |  Ubuntu Server 24.04  |  Zabbix 7.0

Apache  |  Active Directory  |  OSPF  |  pfSense Firewall

# 1. Project Overview

This document details the design, implementation, and verification of a fully functional enterprise branch network lab built using GNS3 and VMware Workstation. The lab simulates a real-world enterprise network environment featuring perimeter security, dynamic routing, VLAN segmentation, centralized identity management, a DMZ web server, and comprehensive network monitoring.

The primary goal of this project is to demonstrate hands-on skills in network administration, system administration, and security — skills directly applicable to entry-level Network Administrator, System Administrator, and IT Support Engineer roles.

## 1.1 Objectives

- Design and implement an enterprise-grade network topology in GNS3

- Configure pfSense as a perimeter firewall with NAT and security rules

- Implement OSPF dynamic routing with MD5 authentication across three routers

- Configure VLAN segmentation with ACL-based inter-VLAN security policy

- Deploy Windows Server 2022 with Active Directory, DNS, and DHCP

- Set up an Ubuntu DMZ server running Apache web services

- Implement centralized monitoring using Zabbix 7.0 with email alerting

- Configure SSH access on all routers for secure remote management

## 1.2 Lab Environment

| Component | Details |
| --- | --- |
| Host Machine | Physical PC, 16GB RAM |
| Hypervisor | VMware Workstation Pro 25 |
| Network Simulator | GNS3 with GNS3 VM |
| Router Platform | Cisco 7200 Series (IOS) |
| Switch Platform | Cisco IOU L2 |
| Firewall | pfSense CE 2.8.1 |
| Server OS | Windows Server 2022 |
| Linux OS | Ubuntu Server 24.04 LTS |
| Monitoring | Zabbix 7.0.25 |

# 2. Network Topology

The topology consists of a pfSense firewall at the edge, three Cisco 7200 routers forming the OSPF backbone, managed switches providing VLAN access, Windows Server 2022 as the enterprise domain controller, and Ubuntu Server in a dedicated DMZ.

## 2.1 Device Roles

| Device | Type | Role |
| --- | --- | --- |
| pfSenseFW | pfSense CE VM | Edge firewall, NAT, WAN gateway |
| HQ | Cisco 7200 Router | Head-office transit router, OSPF |
| BR2 | Cisco 7200 Router | Branch core transit router, OSPF |
| BR1 | Cisco 7200 Router | Branch gateway, inter-VLAN routing, DHCP relay |
| LANSW | Cisco IOU L2 Switch | LAN access switch, VLAN 10/20/99 trunk |
| DMZSW | Cisco IOU L2 Switch | DMZ access switch, VLAN 100 |
| DC01 | Windows Server 2022 VM | Domain Controller, DNS, DHCP |
| Ubuntu Server | Ubuntu 24.04 VM | DMZ web server, Apache, Zabbix |

## 2.2 Physical Connections

| From | Interface | To | Interface |
| --- | --- | --- | --- |
| pfSense Cloud | eth1 (VMnet2) | HQ | fa0/0 |
| HQ | fa0/1 | BR2 | fa0/0 |
| BR2 | fa0/1 | BR1 | fa0/0 |
| BR1 | fa0/1 | LANSW | e0/0 |
| BR1 | fa2/0 | DMZSW | e0/0 |
| LANSW | e0/1 | PC1 | e0 |
| LANSW | e0/2 | PC2 | e0 |
| LANSW | e1/1 | PC3 | e0 |
| LANSW | e1/2 | PC4 | e0 |
| LANSW | e2/0 | DC01 Cloud | eth2 (VMnet3) |
| DMZSW | e0/1 | Ubuntu Cloud | eth3 (VMnet4) |

## 2.3 VMware Network Adapter Mapping

| VMnet | Type | Purpose | Connected VM |
| --- | --- | --- | --- |
| VMnet0 | Bridged | pfSense WAN — internet access | pfSense (Adapter 1) |
| VMnet2 | Host-only | pfSense LAN to GNS3 HQ | pfSense (Adapter 2) |
| VMnet3 | Host-only | DC01 to GNS3 LANSW | DC01 (Adapter 1) |
| VMnet4 | Host-only | Ubuntu to GNS3 DMZSW | Ubuntu (Adapter 1) |

# 3. IP Addressing Plan

## 3.1 Subnet Allocations

| Network | Subnet | Purpose | Gateway |
| --- | --- | --- | --- |
| WAN (pfSense) | DHCP from ISP | pfSense WAN link | Physical network |
| HQ-pfSense | 10.0.0.0/30 | pfSense eth2 to HQ fa0/0 | N/A |
| HQ-BR2 | 10.0.1.0/30 | HQ fa0/1 to BR2 fa0/0 | N/A |
| BR2-BR1 | 10.0.2.0/30 | BR2 fa0/1 to BR1 fa0/0 | N/A |
| VLAN 10 (HR) | 192.168.10.0/24 | PC1, PC2 | 192.168.10.1 |
| VLAN 20 (IT) | 192.168.20.0/24 | PC3, PC4 | 192.168.20.1 |
| VLAN 99 (MGMT) | 192.168.99.0/24 | DC01, Management | 192.168.99.1 |
| VLAN 100 (DMZ) | 192.168.100.0/24 | Ubuntu Server | 192.168.100.1 |

## 3.2 Interface IP Assignments

| Device | Interface | IP Address | Description |
| --- | --- | --- | --- |
| pfSense | em0 (WAN) | DHCP | Internet uplink |
| pfSense | em1 (LAN) | 10.0.0.1/30 | LAN to HQ |
| HQ | fa0/0 | 10.0.0.2/30 | To pfSense |
| HQ | fa0/1 | 10.0.1.1/30 | To BR2 |
| BR2 | fa0/0 | 10.0.1.2/30 | To HQ |
| BR2 | fa0/1 | 10.0.2.1/30 | To BR1 |
| BR1 | fa0/0 | 10.0.2.2/30 | To BR2 |
| BR1 | fa0/1.10 | 192.168.10.1/24 | VLAN 10 gateway (HR) |
| BR1 | fa0/1.20 | 192.168.20.1/24 | VLAN 20 gateway (IT) |
| BR1 | fa0/1.99 | 192.168.99.1/24 | VLAN 99 gateway (MGMT) |
| BR1 | fa2/0 | 192.168.100.1/24 | VLAN 100 gateway (DMZ) |
| DC01 | Ethernet | 192.168.99.10/24 | Static — Domain Controller |
| Ubuntu | ens32 | 192.168.100.10/24 | Static — DMZ web server |

## 3.3 DHCP Pools (served by DC01)

| Scope | Range | Gateway | DNS Server |
| --- | --- | --- | --- |
| VLAN 10 — HR Users | 192.168.10.50 — 192.168.10.200 | 192.168.10.1 | 192.168.99.10 |
| VLAN 20 — IT Users | 192.168.20.50 — 192.168.20.200 | 192.168.20.1 | 192.168.99.10 |

# 4. Router Configurations

## 4.1 HQ Router

Interface Configuration

interface FastEthernet0/0

ip address 10.0.0.2 255.255.255.252

ip ospf message-digest-key 1 md5 <password>

ip ospf authentication message-digest

no shutdown

!

interface FastEthernet0/1

ip address 10.0.1.1 255.255.255.252

ip ospf message-digest-key 1 md5 <password>

ip ospf authentication message-digest

no shutdown

OSPF Configuration

router ospf 1

network 10.0.0.0 0.0.0.3 area 0

network 10.0.1.0 0.0.0.3 area 0

default-information originate always

Default Route

ip route 0.0.0.0 0.0.0.0 10.0.0.1

SSH Configuration

hostname HQ

ip domain-name lab.local

crypto key generate rsa modulus 1024

ip ssh version 2

username vasanth privilege 15 secret <password>

line vty 0 4

transport input ssh

login local

SNMP Configuration

snmp-server community VK_Lab@55 RO

snmp-server location GNS3-Lab

snmp-server contact vasanth@lab.local

## 4.2 BR2 Router

BR2 acts as the transit router between HQ and BR1. Configuration mirrors HQ with appropriate interface IPs.

| Interface | IP Address | Connected To |
| --- | --- | --- |
| fa0/0 | 10.0.1.2/30 | HQ fa0/1 |
| fa0/1 | 10.0.2.1/30 | BR1 fa0/0 |

## 4.3 BR1 Router (Branch Gateway)

BR1 is the most complex router — it handles inter-VLAN routing via subinterfaces, DHCP relay to DC01, and DMZ connectivity.

Subinterface Configuration

interface FastEthernet0/1

no ip address

no shutdown

!

interface FastEthernet0/1.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip helper-address 192.168.99.10

!

interface FastEthernet0/1.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.99.10

!

interface FastEthernet0/1.99

encapsulation dot1Q 99

ip address 192.168.99.1 255.255.255.0

!

interface FastEthernet2/0

ip address 192.168.100.1 255.255.255.0

ip access-group DMZ_POLICY in

# 5. Switch Configurations

## 5.1 LANSW — LAN Access Switch

| Interface | Mode | VLAN | Connected To |
| --- | --- | --- | --- |
| e0/0 | Trunk | 10, 20, 99 | BR1 fa0/1 |
| e0/1 | Access | VLAN 10 | PC1 |
| e0/2 | Access | VLAN 10 | PC2 |
| e1/1 | Access | VLAN 20 | PC3 |
| e1/2 | Access | VLAN 20 | PC4 |
| e2/0 | Access | VLAN 99 | DC01 (via cloud) |

## 5.2 DMZSW — DMZ Access Switch

| Interface | Mode | VLAN | Connected To |
| --- | --- | --- | --- |
| e0/0 | Access | VLAN 100 | BR1 fa2/0 |
| e0/1 | Access | VLAN 100 | Ubuntu Server (via cloud) |

# 6. pfSense Firewall Configuration

## 6.1 Interface Configuration

| Interface | Physical NIC | IP | Role |
| --- | --- | --- | --- |
| WAN | em0 | DHCP from physical network | Internet uplink |
| LAN | em1 | 10.0.0.1/30 | Internal network gateway |

## 6.2 Static Route

A static route is configured to direct all internal traffic back to HQ router:

| Destination | Gateway | Interface | Purpose |
| --- | --- | --- | --- |
| 192.168.0.0/16 | 10.0.0.2 | LAN (em1) | Route to all internal VLANs via HQ |

## 6.3 Outbound NAT Rules

Manual outbound NAT is configured to translate all internal traffic to the WAN IP:

| Source | Destination | Translation | Description |
| --- | --- | --- | --- |
| 192.168.0.0/16 | Any | WAN Address | Internal VLANs to internet |
| 10.0.0.0/8 | Any | WAN Address | Transit links to internet |

## 6.4 Firewall Rules

| Rule | Action | Source | Destination | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Allow | Any | Any | Allow all internal traffic out (lab) |
| 2 | Allow | LAN net | DMZ 192.168.100.0/24 | LAN to DMZ access |
| 3 | Block | DMZ net | LAN 192.168.0.0/16 | DMZ cannot initiate to LAN |

# 7. Security Policy (ACLs)

Access Control Lists are applied on BR1 to enforce inter-VLAN security. The policy follows a least-privilege model — each VLAN can only access what it needs.

## 7.1 VLAN Security Policy Summary

| Source VLAN | Destination | Access | Reason |
| --- | --- | --- | --- |
| VLAN 10 — HR | VLAN 20 (IT) | BLOCKED | Department separation |
| VLAN 10 — HR | VLAN 100 (DMZ) | BLOCKED | HR has no need for DMZ |
| VLAN 10 — HR | VLAN 99 (MGMT) | ALLOWED | Needs AD/DNS/DHCP |
| VLAN 10 — HR | Internet | ALLOWED | Normal browsing |
| VLAN 20 — IT | VLAN 10 (HR) | BLOCKED | IT cannot access HR data |
| VLAN 20 — IT | VLAN 100 (DMZ) | ALLOWED | IT manages DMZ servers |
| VLAN 20 — IT | Internet | ALLOWED | Normal browsing |
| VLAN 99 — MGMT | Everywhere | ALLOWED | Admin VLAN — full access |
| VLAN 100 — DMZ | VLAN 20 (IT) | ALLOWED | Respond to IT requests |
| VLAN 100 — DMZ | VLAN 99 (MGMT) | ALLOWED | Respond to admin requests |
| VLAN 100 — DMZ | VLAN 10 (HR) | BLOCKED | DMZ cannot reach HR |
| VLAN 100 — DMZ | Internet | ALLOWED | Updates and outbound |

## 7.2 ACL Configurations on BR1

VLAN10_POLICY (applied inbound on fa0/1.10)

ip access-list extended VLAN10_POLICY

deny   ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

deny   ip 192.168.10.0 0.0.0.255 192.168.100.0 0.0.0.255

permit ip any any

VLAN20_POLICY (applied inbound on fa0/1.20)

ip access-list extended VLAN20_POLICY

deny   ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

permit ip any any

DMZ_POLICY (applied inbound on fa2/0)

ip access-list extended DMZ_POLICY

permit ip 192.168.100.0 0.0.0.255 192.168.20.0 0.0.0.255

permit ip 192.168.100.0 0.0.0.255 192.168.99.0 0.0.0.255

deny   ip 192.168.100.0 0.0.0.255 192.168.10.0 0.0.0.255

deny   ip 192.168.100.0 0.0.0.255 192.168.0.0 0.16.0.0

permit ip any any

# 8. Windows Server 2022 (DC01)

## 8.1 Server Configuration

| Setting | Value |
| --- | --- |
| Hostname | DC01 |
| IP Address | 192.168.99.10/24 |
| Default Gateway | 192.168.99.1 (BR1) |
| DNS | 127.0.0.1 (self) |
| Domain | lab.local |
| VLAN | 99 (Management) |

## 8.2 Active Directory

DC01 is configured as the primary Domain Controller for the lab.local domain. The following organizational structure has been implemented:

- Domain: lab.local

- Organizational Unit: IT DEPARTMENT

- Security Group: IT_Staff

- Domain Users: vasanth, Ben Stokes, Dhanush, Gokul, Madesh, Sandeep, Vignesh

- Domain Computer: CLIENT1

## 8.3 DHCP Scopes

| Scope Name | Network | Range | Gateway | DNS |
| --- | --- | --- | --- | --- |
| VLAN10-Users | 192.168.10.0/24 | 10.50 — 10.200 | 192.168.10.1 | 192.168.99.10 |
| VLAN20-Users | 192.168.20.0/24 | 20.50 — 20.200 | 192.168.20.1 | 192.168.99.10 |

## 8.4 PowerShell Automation Scripts

The following PowerShell scripts are stored in C:\Scripts\ for AD administration:

| Script | Purpose |
| --- | --- |
| BulkCreateUsers.ps1 | Bulk create AD users from CSV file |
| ADUserReport.ps1 | Generate report of all AD users |
| AddUsersToGroup.ps1 | Add multiple users to a security group |
| InactiveUsers.ps1 | Find and disable inactive user accounts |
| UnlockAccounts.ps1 | Unlock locked-out user accounts |
| ServiceCheck.ps1 | Check status of critical services |
| UpdateDepartment.ps1 | Bulk update user department attributes |

# 9. Ubuntu Server 24.04 (DMZ)

## 9.1 Server Configuration

| Setting | Value |
| --- | --- |
| Hostname | ubuntu-server |
| IP Address | 192.168.100.10/24 |
| Default Gateway | 192.168.100.1 (BR1 fa2/0) |
| DNS Servers | 192.168.99.10, 8.8.8.8 |
| VLAN | 100 (DMZ) |
| Interface | ens32 |

## 9.2 Network Configuration (netplan)

network:

version: 2

ethernets:

ens32:

dhcp4: no

addresses:

- 192.168.100.10/24

routes:

- to: default

via: 192.168.100.1

nameservers:

addresses:

- 192.168.99.10

- 8.8.8.8

## 9.3 Apache Web Server

Apache2 is installed and serves a custom corporate DMZ web page at http://192.168.100.10. The page displays server hostname, IP address, VLAN assignment, and admin details.

- Service: apache2

- Port: 80 (HTTP)

- Document root: /var/www/html/

- Status: enabled and running on boot

## 9.4 Zabbix Monitoring Stack

Ubuntu hosts the complete Zabbix 7.0 monitoring stack including server, frontend, and agent:

| Component | Version | Port | Purpose |
| --- | --- | --- | --- |
| Zabbix Server | 7.0.25 | 10051 | Monitoring engine |
| Zabbix Frontend | 7.0.25 | 80/443 | Web dashboard (Apache) |
| Zabbix Agent | 7.0.25 | 10050 | Local host monitoring |
| MySQL | 8.0 | 3306 | Zabbix database backend |

# 10. Zabbix Monitoring Configuration

## 10.1 Monitored Hosts

| Host | Template | Interface | IP:Port |
| --- | --- | --- | --- |
| Zabbix server | Linux by Zabbix agent, Zabbix server health | Agent | 127.0.0.1:10050 |
| Ubuntu-Server | Linux by Zabbix agent, ICMP Ping | Agent | 192.168.100.10:10050 |
| DC01 | Windows by Zabbix agent, ICMP Ping | Agent | 192.168.99.10:10050 |
| HQ-Router | Cisco IOS by SNMP | SNMP | 10.0.0.2:161 |
| BR2-Router | Cisco IOS by SNMP | SNMP | 10.0.1.2:161 |
| BR1-Router | Cisco IOS by SNMP | SNMP | 10.0.2.2:161 |

## 10.2 SNMP Configuration

All routers are configured with a custom SNMP community string for security:

snmp-server community VK_Lab@55 RO

snmp-server location GNS3-Lab

snmp-server contact vasanth@lab.local

## 10.3 Email Alerting

| Setting | Value |
| --- | --- |
| SMTP Server | smtp.gmail.com |
| SMTP Port | 587 |
| Security | STARTTLS |
| Alert Email | vasanthkumarvk2855@gmail.com |
| Minimum Severity | Warning |
| Recovery Alerts | Enabled |

## 10.4 Alert Types Configured

- Host down / unreachable

- Router restart detected (uptime < 10 minutes)

- High memory utilization (> 90% for 5 minutes)

- Apache web service down

- Zabbix agent unavailable

- Interface high error rate

- Recovery notifications when issues resolve

# 11. Verification and Testing

## 11.1 OSPF Verification

OSPF neighborship was verified on all routers. All neighbors achieved FULL state with MD5 authentication:

| Router | Neighbor | State | Interface |
| --- | --- | --- | --- |
| HQ (1.1.1.1) | BR2 (2.2.2.2) | FULL/BDR | fa0/1 |
| BR2 (2.2.2.2) | HQ (1.1.1.1) | FULL/BDR | fa0/0 |
| BR2 (2.2.2.2) | BR1 (3.3.3.3) | FULL/DR | fa0/1 |
| BR1 (3.3.3.3) | BR2 (2.2.2.2) | FULL/DR | fa0/0 |

## 11.2 End-to-End Connectivity Matrix

| Source | Destination | Expected | Result |
| --- | --- | --- | --- |
| PC1 (VLAN10) | 192.168.10.1 (BR1 GW) | PASS | PASS |
| PC1 (VLAN10) | 192.168.99.10 (DC01) | PASS | PASS |
| PC1 (VLAN10) | 192.168.20.50 (PC3) | BLOCK | BLOCK |
| PC1 (VLAN10) | 192.168.100.10 (Ubuntu) | BLOCK | BLOCK |
| PC1 (VLAN10) | 8.8.8.8 (Internet) | PASS | PASS |
| PC3 (VLAN20) | 192.168.100.10 (Ubuntu) | PASS | PASS |
| PC3 (VLAN20) | 192.168.10.51 (PC1) | BLOCK | BLOCK |
| Ubuntu (DMZ) | 192.168.99.10 (DC01) | PASS | PASS |
| Ubuntu (DMZ) | 192.168.10.51 (PC1) | BLOCK | BLOCK |
| Ubuntu (DMZ) | 8.8.8.8 (Internet) | PASS | PASS |
| DC01 (MGMT) | 192.168.100.10 (Ubuntu) | PASS | PASS |
| DC01 (MGMT) | 8.8.8.8 (Internet) | PASS | PASS |

## 11.3 DHCP Verification

All PCs successfully received IP addresses from DC01 via DHCP relay on BR1:

| PC | IP Received | VLAN | DHCP Server |
| --- | --- | --- | --- |
| PC1 | 192.168.10.51/24 | VLAN 10 | 192.168.99.10 (DC01) |
| PC2 | 192.168.10.52/24 | VLAN 10 | 192.168.99.10 (DC01) |
| PC3 | 192.168.20.50/24 | VLAN 20 | 192.168.99.10 (DC01) |
| PC4 | 192.168.20.51/24 | VLAN 20 | 192.168.99.10 (DC01) |

## 11.4 Service Verification

| Service | Host | Test Method | Status |
| --- | --- | --- | --- |
| Active Directory | DC01 | Domain join, user auth | Operational |
| DNS | DC01 | nslookup lab.local | Operational |
| DHCP | DC01 | ip dhcp on VPCS | Operational |
| Apache Web Server | Ubuntu | http://192.168.100.10 | Operational |
| SSH (HQ) | 10.0.0.2 | PuTTY from DC01 | Operational |
| SSH (BR2) | 10.0.1.2 | PuTTY from DC01 | Operational |
| SSH (BR1) | 10.0.2.2 | PuTTY from DC01 | Operational |
| Zabbix Monitoring | Ubuntu | http://192.168.100.10/zabbix | Operational |
| NAT/Internet | pfSense | ping 8.8.8.8 from all hosts | Operational |

# 12. Known Issues and Limitations

| Issue | Impact | Workaround |
| --- | --- | --- |
| GNS3 cable instability on restart | Some router links drop after GNS3 VM restart | Redraw affected cables after each restart |
| Zabbix and Apache on same host | Zabbix WebUI unavailable when Apache is stopped | Architectural limitation in single-VM setup |
| pfSense on bridged network | WAN IP changes with physical network | Lab environment limitation; static WAN not needed |

# 13. Skills Demonstrated

## Networking

- Enterprise network design and topology planning

- OSPF dynamic routing with MD5 authentication

- VLAN configuration and 802.1Q trunking

- Inter-VLAN routing using router-on-a-stick

- ACL design and implementation

- NAT/PAT configuration on pfSense

- DHCP relay using ip helper-address

- SSH hardening on Cisco routers

- SNMP monitoring configuration

## System Administration

- Active Directory Domain Services deployment

- DNS and DHCP server configuration on Windows Server 2022

- PowerShell scripting for AD automation

- Ubuntu Server administration and service management

- Apache web server deployment and configuration

- Netplan network configuration on Ubuntu

## Security

- pfSense perimeter firewall configuration

- DMZ design and implementation

- VLAN-based network segmentation

- ACL-based access control policy

- OSPF MD5 authentication

- SSH key-based authentication on routers

## Monitoring & Operations

- Zabbix 7.0 deployment with MySQL backend

- Multi-platform monitoring (Linux, Windows, Cisco SNMP)

- Email alert configuration with Gmail SMTP

- Custom trigger creation for service monitoring

- Dashboard creation for infrastructure overview

Contact Information

Vasanth Kumar R

Email: vasanthkumarvk2855@gmail.com

GitHub: github.com/vasanth-kumar-vk

LinkedIn: linkedin.com/in/vasanth-kumar-vk55

Certification: Cisco Networking Basics (February 2026)